District 12-5 Bridge Crew Trailer Bid Specs

The following specs are for an enclosed heavy duty utility trailer for the Westmoreland County Penn DOT Bridge Crew. This unit must be a new trailer model year 2015 or newer from a manufacturer or a manufacturer’s authorized dealer. Successful bidder will be responsible for delivering the finished unit to 144 Donohoe Road Greensburg Pa 15601 no later than 60 days after receiving a fully executed PO. PENNDOT Reserves the right to inspect the finished unit to ensure it meets the specifications below prior to releasing final payment. Specifications below are requirements, no substitutions are permitted.

Warranty:

- This trailer shall have a minimum of a 2 year Bumper to Bumper Warranty covering everything inside and out, except brake shoes, drums/rotors and tires.

- This warranty shall cover labor and parts without deductibles.

- All warranty policies shall be from the OEM and repairs shall be made at any OEM warranty repair facility.

- Parts replaced under this warranty will be of OEM quality or higher.

- Successful bidder must present a warranty card during unit delivery

On Board equipment Requirements:

- The trailer shall include the proper forms to apply for a Pennsylvania title and license. These forms will include: A detailed invoice listing all equipment, PO number, VIN numbers and price. All title papers shall be properly prepared and executed. The Application for title, Form MV-1 shall indicate the name and address to be supplied by the District 12-5 Equipment Manager prior to delivery.

- Trailers shall be certified to meet or exceed requirements to obtain a Pennsylvania License.

- Trailer shall have a valid Pennsylvania state inspection sticker.

- Trailer shall have completed the manufacturer’s recommended pre-delivery service.

- Trailer shall have the vehicle Manufacturer’s model name, model number and GVW stated on a decal or placard attached to the vehicle.

- Trailer shall be free from all dealer signs/emblems.

Delivery

- Successful bidder will be responsible to deliver the finished trailer to the address below.

- Successful bidder will assume all costs associated with delivery and should bid accordingly.

- Delivery must be scheduled at least three days in advance to ensure End Users are available for the final delivery inspection.

- Delivery must be made to the address below no later than Sixty(60) calendar days after successful bidder receives a fully executed PO.

  - PENNDOT District 12-5

  - 144 Donohoe Road

  - Greensburg, PA 15601

Body

- The dimensions of the trailer are 20 feet in length and 8.5 feet in width.

- The exterior of the trailer will be screwless aluminum panels.

- The exterior paint color of the trailer will be PENSKE YELLOW.

- The roof of the trailer will be painted WHITE.

- The frame of the trailer including the hitch will be painted the color BLACK.

- Red and White DOT conspicuity tape will be placed all the way around the bottom perimeter of the trailer.

- The exterior roof of the trailer will be reinforced to allow workers to stand on top and remove items attached to the ladder racks.

- A 2 foot high panel of diamond plate will be mounted across the front of the trailer to resist stone and debris damage.

- The front two corners of the trailer will be covered in diamond plate to resist damage.

- On the right side of the trailer an 18ft. long roll out RV awning COLOR GREY will be mounted centered on the trailer.

- On the right side of the trailer a 48” RV man door with interior screen door will be mounted 3ft back from the front of the trailer to the center of the door. The RV door will be equipped with a paddle latch and cargo vise latch w/key and tumbler locks. This door must be able to open with the RV awning rolled out.

- The rear of the trailer will be equipped with two barn style doors. The doors will have cargo vise latch with key tumbler lock keyed the same as the RV door. The interior of the doors will be clad in 23/32 plywood.

- Two 14”x14” manual open roof vents. Each vent will installed 6 feet from the front and 6 feet from the rear of the trailer.

Frame

- The trailer will have tandem axles rated at 6,000lbs per axle.

- Tires will be 235/80R16E with an 8 bolt pattern mounted on aluminum MOD wheels. NO SPARE TIRE NEEDED.

- The drawbar will have an 18inch extended hitch with a height adjustable 2 5/16” Pintle.

- This trailer will be equipped with hydraulic surge brakes NOT electric trailer brakes.

- An 8,000lb manual front jack will be mounted to the drawbar.

- Diamond plate will mounted to the drawbar a-frame from the front of the trailer to the tongue jack. The diamond plate will be used as a platform to access the ladder.

Exterior Electrical

- All red and amber DOT required trailer lighting will be LED units.

- All brake and turn signal lights will be LED units. An intermediate turn signal will be mounted between the tandem axles 3 inches above the fenders.

- Turn signal and Brake lights will be a separate units.

- A Grote 7-pin trailer plug for the trailer lights, turn signals and brake lights.

- One 110V 30amp twist lock receptacle in weather resistant housing will be mounted on the exterior of the trailer 1ft. to the left of the RV man door 1ft above the interior floor height.

Ladders and Roof racks

- The exterior roof will have a total of four aluminum ladder rack bars mounted at equal spacing starting 1 foot in from the front corner of the trailer and 1 foot in from the rear corner of the trailer.

- Starting at the front working toward the back 3 standard racks will be mounted and each rack will have a min. combined capacity of 250lbs.

- The rear most rack will have an offset rack so the cross bar is flush with the rear of the trailer and also have a min. combined capacity of 250lbs.

- An aluminum ladder will be mounted to the exterior front of the trailer in the center. The ladder will run from the drawbar a-frame to the roof and the ladder handrail will extend to the ladder rack.

Interior

- The interior height of the trailer 8 feet – 6 inches.

- The interior ceiling will be insulated and finished in smooth WHITE vinyl panels.

- Interior walls will be covered in 23/32 plywood painted white.

- The interior floor throughout will be covered in rubber coin pattern flooring color BLACK.

- An aluminum clad workbench 28 inches deep will run the entire width of the front wall of the trailer. DO NOT INSTALL the workbench but fabricate and provide all mounting hardware necessary to install.

- Metal(Aluminum or steel) cabinets and doors will be installed on the front wall of the trailer 58 inches to the bottom of the cabinet from the finished floor height and extend up to the finished ceiling . The cabinets will run the full width of the trailer on the front wall.

- Cabinet doors will have locks to prevent the doors from opening during transit.

Interior Electrical

- From the rear of the 110V 30amp plug wire 10-3 stranded PVC coated wire to a 6 slot breaker panel mounted 60 inches from the finished floor height.

- The breaker panel will have five 20amp breakers and one 25 amp breaker.

- All wiring from the breaker panel to the outlets will be stranded 12-3 PVC coated wire.

- All wiring from the breaker panel to the lights will be stranded 14-3 PVC coated wire.

- All outlets and switches will be installed in metal gang boxes with metal face plates.

- A total of nine duplex outlets will be installed on the interior of the trailer.

- Three 20amp outlets will be mounted 48 inches above the finished floor height along the front wall evenly spaced left to right.

- Two 20amp Duplex outlets will be mounted on the left side of the side RV door 12 inches from the door edge and 36 inches from the floor and the second mounted inline above 48 inches from the floor.

- Two 20 amp duplex outlets will be installed in a double gang box on the left wall at the rear of the trailer 18 inches forward from the rear doors and 42 inches from the finished floor height.

- Two 20 amp duplex outlets will be installed in a double gang box on the right wall at the rear of the trailer 18 inches forward from the rear doors and 42 inches from the finished floor height.

- Two 6ft. long 10in. wide flush mount hard wired LED shop lights with a wraparound diffuser will be mounted to the interior ceiling.

- The first light will be centered between the two roof vents down the center of the trailer front to back.

- The second fixture will mount 4ft back from the front interior wall of the trailer and be mounted sideways from left side to the right side centered between the two walls.

- All shop lights will be switched from a single pole switch located on the right side of the RV door 12 inches from the edge of the door and 52 inches from the finished floor.

*Any questions please email chinman@pa.gov