# GENERAL SPECIFICATIONS

PENNSYLVANIA GAME COMMISSION

FORESTRY DIVISION

FORESTRY MOWING CONTRACT

(HAND CUTTING)

- SCOPE OF WORK:

- The Pennsylvania Game Commission, Forestry Division, requires the services of an Operator to cut interfering woody vegetation five feet high up to and including stems six inches in diameter at breast height (DBH) as described below on 257 acres on SGL 204, Hebron Township, Potter County.  Services shall include, but are not limited to, the furnishing of all materials, labor, equipment, tools, superintendence, miscellaneous items, and performing all work necessary to complete the project to the satisfaction of and subject to the approval of the Commission.

- Questions regarding any technical aspects of this bid should be directed to Regional Forester Gary Glick, at telephone no. (570) 398-4744 Ext. 30310.  Questions specific to the bidding process should be directed to the PGC Purchasing and Procurement Division at telephone no. (717) 787-6594.

# CONTRACT TASKS:

  - Project Location(s): N 41.8604227 W -78.0849119

    - Approximately 257 acres on State Game Lands 204 will require the mechanical mowing or hand cutting of interfering woody vegetation under this contract. See attached project and location maps.

Approximate stems per acre to be felled include:

| Block Number | Acres | Total number of stems per acre to cut | Stems per acre that are 6 to 8 inches dbh |
| --- | --- | --- | --- |
| 1 | 175 | 770 | 8 |
| 2 | 51 | 1440 | 32 |
| 3 | 31 | 930 | 1 |

    - Counties Included:

Potter County, Hebron Township

- SITE INSPECTION AND ASSISTANCE:

- An optional site inspection tour of the project site is scheduled for August 8, 2022. The meeting location is at the intersection of Fishing Creek Road and SR 6 across from the Dollar General store.  (N 41.7842036 W -78.1609329)

- Site inspection will provide prospective bidders the opportunity to view the site and evaluate conditions specific to the project, including vegetation for control, topography, site challenges (rock, downed logs, density of reserved trees, etc.)

- COMMISSION PERSONNEL

- The Commission will furnish personnel to supervise and direct the operation in the following capacities:

- PROGRAM SUPERVISOR - The Chief of the Forestry Division in the Bureau of Wildlife

Habitat Management is responsible for the overall operation of the vegetation control program and represents the Commission in settling minor contractual matters.

- FIELD CONTRACT COORDINATORS - These persons are Regional Foresters or their designee who serve as the Commission's field liaison with the Operator and are responsible for reviewing and approving/disapproving the operations plan and conducting quality control checks on the equipment and performance during the operations.  This person is responsible for coordinating the activities of all people working on the contract within a region and communicating with the Operator's project supervisor concerning any operational problems or changes in plans.  He/she also provides daily reports of the operation to the program supervisor

- EQUIPMENT SPECIFICATIONS:

- Hand cutting may be done by chainsaw or with brush cutters on a weedeater type of platform.  me sections of the blocks and there are no skid trails.  Hand cutting is permissible in all sections of all blocks.

- All machines used on this project must have a minimum 95 horsepower.

- The Commission reserves the right to reject any piece of equipment that in its opinion will not

safely and adequately complete the job.

- CONTRACT REQUIREMENTS:

- Mowing Blocks 1 and 2 are designated with orange paint marks.  These areas are located in timber sale and herbicide areas where other colors of paint may be also observed. All trees marked with any color of paint are reserved regardless of diameter and regardless of paint color. Block 3 is designated by the deer exclosure fence and includes the entire area inside the fence.

- In Blocks 1 and 2, all stems five feet high and up to and including stems 8 inches DBH must be felled. All trees within 10 feet of any above ground gas line are also reserved from cutting.

- In Block 3, only American beech, black birch, and striped maple stems 5 feet high and up to 6 inches in diameter must be cut.  All other species of trees in Block 3 are reserved.

- Woody material felled with clean cuts must have stumps that are parallel to the ground surface and shall not exceed six (6) inches in height measured on the side next to the highest ground or the diameter of the stump, whichever is smaller, except when in the opinion of the Field Contract Coordinator, said height is impractical.

- All tops and debris must be pulled back at least 20 feet from the road ditches, tail drains, trails, herbaceous openings, streams, utility right of way, above-ground pipelines, and any color of blazed tree.  Material that cannot be directionally felled or dragged away from these features must be lopped down to sections less than 8 feet and length and tossed aside to keep the corridors clear.

- Above ground gas pipelines are present in each of the cutting blocks.  These are active lines and extreme caution must be used not to damage the pipes or drop debris on them.  Do not cut any trees within 10 feet of above ground gas pipelines.

- The Operator shall exercise care and caution in all operations to prevent damage to all trees not specified for treatment.

- Damage to trails, roads, streams, or utility rights-of-way caused by the Operator’s equipment must be repaired by the Operator at their expense.

- Any trash resulting from the Operator’s operations must be removed from the area and properly disposed.

- The Operator shall not block any roads or trails in the area during performance of this contract. Old skids trails that small trees are growing on must also be kept clear of debris and treetops, including debris from the trees that are cut off of the trail surface.

- The Operator shall not in any way hinder the progress of any Timber Sale Contracts in these areas.

- All labor, equipment, tools, etc., needed to complete contracted projects are to be provided by the Operator.

- Timber Damages – when in the opinion of Field Contract Coordinator, damage to the residual stand becomes excessive, the Operator shall pay the Commission a fair base current value determined by the Field Contract Coordinator per unit of volume. If this value for damage due to Operator’s carelessness or negligence is less than $10.00 per tree, then a minimum charge of $10.00 per tree will be made whether the tree is commercial, non-commercial, merchantable, or non-merchantable.

- Operator will provide their own lock for use on each SGL gate the operator intends to use to access the site; the key or combination must also be provided to the PGC Regional Forester.  Gates must be kept closed at all times and must be locked when the operator is not on site. Operator will be held responsible for repairing any damage to PGC gates caused by the operator.

VII.     OPERATIONS AND NON-WORK DAYS

- No woods operation will be permitted on the following days:

      - All Saturdays in the months of October, November, December, and January and any Sundays open to hunting.

      - The opening day and all Saturdays during spring turkey season, including the youth spring gobbler hunt. (Saturdays April 22, 2023 – May 30, 2023)

      - All the days of the general statewide bear season (November 19, 2022 – November 22, 2022)

      - The entire 2 weeks of the general firearms deer season (November 26, 2022 – December 10, 2022)

      - The opening day of the flintlock muzzleloader season (December 26, 2022)

      - No woods operation will be permitted from March 1 thru April 15 (SPRING BREAK-UP DATES without the permission of the forester-in-charge.

B. The contractor, employees, and sub-contractors may not hunt to, from, or at the job site when the work location is beyond a gated road that is closed to public traffic.  The contractor may travel to and from the work location on no-work days to check on the condition of equipment.

- The contractor is responsible for the control and collection of fluids leaking from any equipment used on the site. The contractor must have a spill containment and cleanup kit appropriate for the equipment being used.

- It is unlawful to be present on State Game Lands from November 15 through December 15 inclusive when not engaged in lawful hunting or trapping and fail to wear a minimum of 250 square inches of daylight fluorescent-orange material on the head, chest, and back combined, or in lieu thereof, a hat of the same colored material.  The material shall be worn so it is visible in a 360-degree arc.

# INSURANCE REQUIREMENTS:

- The contractor will provide Workmen’s Compensation insurance sufficient to cover all the employees of contractor working to fulfill this contract.

- Comprehensive General Liability Insurance, including bodily injury and property damage insurance, to protect the Commonwealth and the contractor from claims arising out of the performance of the contract.  The amount of bodily injury insurance shall not be less than $300,000.00 for injury to or death of persons per occurrence.  The amount of Property Damage Insurance shall not be less than $300,000.00 per occurrence.

- Such policies shall name the Commonwealth as an additional insured.  Prior to the commencement of work under this contract, the contractor must provide the Commonwealth with current Certificates of Insurance.  These Certificates shall contain a provision that coverages afforded under the policy shall not be canceled or changed until at least thirty (30) days prior written notice has been given to the Commonwealth.

- The Commonwealth shall be under no obligation to obtain such certificates from the Operator(s). Failure by the Commonwealth to obtain the certificates shall not be deemed a waiver of the Operator’s obligation to obtain and furnish certificates. The Commonwealth shall have the right to inspect the original insurance policies.

- HOLD HARMLESS:

- The contractor shall be responsible for and agrees to indemnify and hold harmless the Commonwealth from and against damages to property or injuries (including death) to any persons and other losses, damages, expenses, claims, demands, suits, and actions by any party against the Commonwealth in connection with the work performed by the contractor.

# BIDDING

- RECEIVING AND OPENING BIDS - The bid shall be awarded on a price per acre basis.    Bids must be submitted via the PA Supplier Portal, to be found at www.pasupplierportal.state.pa.us  Faxed bids and mailed bids will NOT be accepted.

- Bids will be publicly opened on Aug 22, 2022 at 2:00 PM., at the Pennsylvania Game Commission, 2001 Elmerton Ave., Harrisburg, PA 17110-9797.

- The price per acre shall include all labor, equipment, tools, insurance, materials disposal and all items necessary for the completion of the project.  Prices bid shall be in dollar and cents only.

B.  BID RESULTS - Bidders can obtain results at www.emarketplace.state.pa.us/bidtabs.aspx

- The posted results will contain the information of the apparent bidders, and all bids are under review until final award of the purchase order.

- Quantities herein are only estimates and may increase or decrease dependent upon the needs of the Commission.  Operator shall be paid at the unit price bid for actual services performed.

- The Commission reserves the right to reject any or all bids and award contracts as it determines to be in the best interest of the Commission.

- PERFORMANCE SECURITY:

  - As further guarantee of faithful performance of the conditions set forth in this contract, the Contractor hereby agrees to furnish the Commission with Certified Check, Savings Account, Irrevocable Letter of Credit, or other security acceptable to the Pennsylvania Game Commission, in the sum of $5,000.00.

    - ANY SECURITY POSTED MUST HAVE AN EXPIRATION DATE A MINIMUM OF ONE YEAR BEYOND THE EXPIRATION DATE OF THE CONTRACT.

  - Performance Security is due no later than 30 days after written notice of the bid award.  Time is of the essence with regard to the provision of Performance Security.  At the option of the PGC, failure to provide an acceptable Surety within 30 days will result in cancellation of the contract and the PGC may declare the bidder ineligible on future projects.

  - It is further agreed that any performance bond guarantees the complete and faithful performance of this contract and full compliance with the conditions set forth herein, particularly as to construction repairs, restorations or other conditions to be performed and the Principal and Surety shall in no way be released or either of them, their executors, administrators, successors or assigns from their liability hereunder, notice to the Surety or Sureties of any such repair involving construction restoration and conditions to be performed, and extension or forbearance hereof being hereby waived.

  - Failure to comply with any of the conditions of the contract herein mentioned shall result in cancellation of the agreement and forfeiture of the performance bond to cover damages sustained by the Commission.  The Commission may declare the Contractor ineligible to bid on future contracts.

# OPERATOR REFERENCES:

- After the bid opening and prior to awarding of the contract, the Commission has the right to request 3 references (names, addresses, and telephone numbers) of similar work performed by the Operator within the previous two (2) years as proof of qualifications to perform the work involved in this contract.  Similar work is defined here as proof that the bidder has successfully completed at least 150 acres of machine and/or hand cut mowing in a forested environment in a single calendar year.

# CONTRACT TERM:

- The Contract shall commence upon execution and receipt of the purchase order and continue until June 23, 2023.

# PAYMENT TERMS:

- Payment shall be on a reimbursement basis for actual acres completed as measured by a Forestry Division employee.  A block will be deemed complete when all items listed in the specifications have been met for that block.  Invoice must state the number of acres properly treated.

- On line items larger than 100 acres, the Operator may invoice for partial payment. The Field Contract Coordinator must be in agreement with the Operator, and the Program Supervisor must authorize the partial payment.

# INVOICES:

- Original invoices must be sent to:

Commonwealth of PA – PO Invoices

P.O.Box 69180

Harrisburg, PA 17106

- A copy of all invoices must be sent to:

Gary J. Glick

Pennsylvania Game Commission

PO Box 5038

Jersey Shore, PA 17740

C.  All invoices must have the Purchase Order Number, as well as the Operator’s SAP Vendor Number on the invoice.  The name and address listed on the purchase order must also be on each invoice.  Failure to provide this information may result in a delay of payment.

    - Please Note: Vendors are reminded to NOT include employer identification numbers, Social Security Numbers, bank account information, or other personally identifiable information on their invoices. That information is uniquely tied to your SAP Vendor Number and, for security purposes, should not be explicitly stated on an invoice.

- PURCHASE ORDER(S) TERMINATION

- The Commonwealth may terminate the Purchase Order(s) for its convenience if the Commonwealth determines termination to be in its best interest.  Operator shall be paid for work completed.

Attachments:

Attachment A: Project Maps