BLOOMSBURG UNIVERSITY

SPECIFICATIONS					25-PASSENGER SHUTTLE BUS SPECIFICATIONS

Please   box if specification is not met and provide information on alternative that meets or exceeds what is specified.

	GVWR	14,050 lbs.	E-450 Super Duty.

	Wheel Base	176” minimum.

	Front Axle	4,600 lbs.

	Rear Axle	9,450 lbs.

	Front Springs	Heavy Duty.

	Rear Springs	Heavy Duty.

	Shock Absorber	Heavy Duty.

	Axle Ratio	4.10 ratio.

	Engine Diesel	V-8 6.0L minimum with block heater

	Transmission	4-speed electronic with overdrive, heavy duty transmission with extra cooler and brake retarder – Telma or approved equal.

	Fast Idle Solenoid	Manual with monitor lights.

	Front Heater	OEM with two (2) extra defroster fan passenger front and driver’s front.

	Rear Heater	35,000 BTU with 2-speed fan with heater shut-off valves for summer use.

	A/C Front	OEM with dual compressors.  14,000 BTU’s.

	A/C Rear	67,000 BTU with its own compressor and three fan condenser.  Bus A/C must have dual compressors.

	Cruise Control with tilt wheel.

	Backing Alarm	Audible, activated when in reverse gear.

	Tires	LT 225-85-R16.

	Wheels	Steel with bolt on wheel covers, stainless steel, or polished aluminum.

	Steering	Power.

	Brakes	4-wheel disc brakes with anti-lock and Telma brake retarder.

	Parking Brake	Foot operated, cable activation to rear brakes.

	Alternator	200 amp with volt meter.

	Batteries	Two each heavy-duty with slide out tray.

	Air Filter	Dry type.

	Oil Filter	Spin-on disposable.

	Fuel filter	With drain for water removal.

- Fuel	Diesel with label at fill cap (Diesel Fuel only).

	Fuel tank	35 gallon or greater

	Instruments	Digital clock mounted in control panel.  Warning system - low oil, high water temperature and buzzer, Kysor shut down system.

	Driver’s Safety	Air bag / shoulder harness and lap belt.

	Interior Mirrors	Flat rear view mirror plus convex mirror for viewing passenger compartment.

	Exterior Mirrors	Convex crossing mirror for viewing front of bus area.  Flat mirrors and round convex mirrors with rivnuts and plastic spacers to reduce vibration.

	Horns	Dual electric.

	Hour Meter	Installed in control panel.

	Hood Release	Inside.

	Front Bumper	OEM with rubber or vinyl inserts.

	Rear Bumper	Wrap-around bumper - Romeo rim steel construction, with heavy-duty rubber.

	Headlights	Dual halogen.

	Directional Lights	Front amber with mid-ship turn signals.  Rear red lens.

	Center Brake Light	High mounted rear center brake light for added safety.

	Clearance Lights	Armored marker lights.

	Back-Up Lights	Dual clear.

	Bus Body	Minimum length 305”

	Bus Flooring  	Metal underbelly and insulated floors with blown in foam.

	Seating Capacity	25 adult passenger with grab rails on aisle side.

	Passenger Seat Cover	All Sherpa-Level 3 upholstery cloth/fabric material - maroon in color

	Drivers Seat	Freedman CL67 Air suspension Driver seat with optional flip arms. Cloth upholstery/fabric material -  Maroon in color

	Entrance Door	Electric with hooded exterior light.

	Driver’s Door	OEM.

	Emergency Exits	Window exits.

	Tinted Windows	Tinted for air conditioning.

	Entrance Lighting	Length of the inside of bus interior lights come on when doors open.

	Double Door Opening	36” electric door.

	Exhaust System	Routed to street side discharge.

	Audio	Deluxe AM/FM digital seek and scan stereo radio with CD player and four speakers.

	Storage Area	In overhead compartment driver’s area.

	Interior Panels	Vinyl on plastic sidewalls.  Fabric ceiling panel.  Ceiling height minimum 76.5”

	Floor Covering	Gray rubber.

	Stanchions	On entrance door side padded.

	Stair Well Rails and Entrance Grab Rails parallel to steps on both sides plus Vertical Grab Rails.

	Safety Equipment	Safety kit first aid plus 5 lb. fire extinguisher.

	Exterior Paint	Bright white, clear coated.  Trim color - maroon 6” strip midway on sides of bus with 1” gold strip above and below.

	Lettering	Bloomsburg University.  Front cap and rear cap in maroon.  5” letters, ALL CAPS.

	Running Board	Driver’s side only.

	Spare Tire	Mounted on wheel and loose inside vehicle upon delivery.

	Mud Flaps	Front and rear.

	Insulation	Full body.

	Undercoating	Body and cab fully undercoated.

	Fastening Components	Components such as air conditioning evaporator, stanchions, grab rails, etc. are fastened directly to the steel roll cage members or by using steel plates and cross-members attached directly to the steel roll cage, or honeycomb fiberglass FMVSS approved equal.

	Price should be all inclusive – Title, Tags and Transfer.  Delivery of new vehicle to Maintenance Garage (Buckingham Maintenance Center).