Appendix K

Converged Network Standards

RFP 2 Commonwealth Enterprise Network (EN) and Shared Services

RFP NUMBER

6100039273

# Converged Network Standards

Contents

Overview	2

Definitions	2

Commonwealth Point-of-Presence (CPOP) Locations	2

Provider Responsibilities	3

Critical Sites	3

Telecommunication Service Priority (TSP)	3

Service Delivery Standards	3

Authentication	3

Network Time Protocol (NTP)	3

Interconnection Standards	3

Layer 1 – Physical Layer	4

Layer 2—Data Link Layer	5

Layer 3—Network Layer	5

IPv4 Routing	5

DSCP Standards	6

Layer 3+	6

Perimeter Control – Commonwealth Security Zones	6

Perimeter Control – Service Provider Network	6

Voice Standards	6

Video Standards	7

Video Conferencing	7

General Video Transmission	7

Facilities	7

Environmental Specifications	7

Access to Facilities	8

Monitoring and Surveillance	8

Physical Access	8

Version History

| Date | Version | Modified By / Approved By | Section(s) | Comment |
| --- | --- | --- | --- | --- |
| 11/25/2015 | 1.0 | Dale Kline | All | Initial Version |
| 12/3/2015 | 1.1 | Dale Kine | Layer 1 CWDM IPv4 Routing added BGP DSCP Standards added Life and Safety and General Voice Standards added encryption paragraph Video Standards new Access to Facilities added Monitoring and Surveillance | Information updated in those sections |
| 01/6/2016 | 1.2 | Dale Kline | DSCP Standards added LMR and 911 Voice, Network Control Routing Protocol, Video | Information updated in those sections |
| 01/20/2016 | 1.3 | Dale Kline | Version History Overview Video Conferencing | Information updated in those sections |
| 1/22/2016 | 1.4 | Dale Kline | CPOP Locations | Information updated in those sections |
| 2/4/2016 | 1.5 | Dale Kline | CPOP Locations Layer 1 General Video Transmission | Information updated in those sections |

Note: Replace this information with your own version history information.

# Overview

As the Commonwealth looks forward to a future of interconnecting disparate networks and converging services, it is important that all organizations abide by a common set of standards.  This document provides a set of network standards for those entities wishing to offer communications and/or application services to the Commonwealth.  This applies to Physical and Virtual Environments. Additionally, all providers must adhere to standards listed in other Commonwealth Informational Technology Policies (ITP).

# Definitions

The following definitions are utilized in this Standards document:

| Term | Definition |
| --- | --- |
| RetireD | This standard is no longer accepted by the Commonwealth. Items will only be maintained on this list for a maximum of 24 months after retirement. |
| CONTAIN | The Commonwealth currently supports this standard but no longer accepts it for new services. (possibly match text to ITP language) |
| Preferred | The Commonwealth currently prefers these service standards. |
| Special | The Commonwealth has approved these standards; however, they are not preferred. They may require special equipment or other special considerations. In some cases, additional cost, which may be forwarded to the requesting entity, would be required to implement such a connection. An example of this is the use of Single-Mode (SM) Fiber vs. Multi-Mode (MM) Fiber; such a scenario involves the purchase of expensive SM optics. |
| Future | The Commonwealth foresees the future use of this standard, but the current infrastructure would require upgrades to support it. Entities may request the use of such standard but should not expect that existing facilities will support it. |
| CPOP | A Commonwealth Point-of-Presence (CPOP) is a point of access for service providers to interconnect with Commonwealth’s network(s). |
| REQUIRED | The Commonwealth currently requires these standards. Such requirements may involve public safety concerns or protection of regulated information (ex: CJIS, HIPPA, PII, etc.). |

# Commonwealth Point-of-Presence (CPOP) Locations

Currently, the Commonwealth provides the following CPOP location(s):

- Group A

  - Commonwealth Technology Center

  - Cameron St. (Limited Space and Environmental)

  - Keystone Building (Limited Space and Environmental)

  - M25E Capital Building (Limited Space and Environmental)

  - Rachel Carson Building (Limited Space and Environmental)

  - Strawberry Square (Limited Space and Environmental)

- Group B – Commonwealth Owned facilities outside of Harrisburg

  - TBD

- Group C – State-Wide Radio

  - Radio to provide list of sites

- Group D – Middle-Mile Backbone Sites (Non Commonwealth)

  - TBD

The Commonwealth reserves the right to modify the list of CPOP locations with reasonable notification to all entities currently providing services through any locations changing status.

# Provider Responsibilities

## Critical Sites

Commonwealth-identified critical sites require redundant connections that are physically diverse.  Organizations wishing to provide services to or from locations defined as critical sites must, upon Commonwealth request, provide end-to-end physical and logical path details to ensure that diversification is maintained.  If diversification is not provided by the service provider, the provider shall support the coordination with the Commonwealth, and where appropriate other service provider(s), necessary for the Commonwealth to obtain such diversity through multiple carriers.

## Telecommunication Service Priority (TSP)

All organizations providing common carrier services to the Commonwealth must support the federal TSP program.  If a provider’s service does not qualify for the TSP program, the provider must notify the Commonwealth as such.

## Service Delivery Standards

### Authentication

The Commonwealth considers integration with Commonwealth authentication systems to be preferred.  Providers, whose services include a web-based management or reporting functionality or allow for some form of Commonwealth management of devices, should seek solutions that can integrate with the Commonwealth’s infrastructure for authentication.

### Network Time Protocol (NTP)

Providers must maintain reliable NTP synchronization with all devices to ensure that logging is accurate.  The Commonwealth considers a minimum of Stratum 2 to be Preferred for the time source.

# Interconnection Standards

Included are the standards for interconnection with the Commonwealth network.  Any desire to connect with a deviation from these standards will be dealt with on a case-by-case basis.  In such cases, every effort will be made to conform to the standard or modify the standards list to include another connection method that can be widely utilized.  The Commonwealth will periodically update the standards for connections with reasonable notification to all entities currently affected by a status changing to Contain or Retired.

## Layer 1 – Physical Layer

| Technology | Speed | Status | CPOP Group(s) | Media | Standard(s) | Fiber Size(s) | Connector(s) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Fiber | Any | Contain | N/A | Any | Any | Any | ST, FC |
| Fiber | Any | Special | N/A | Any | Any | Any | SC |
| Fiber | Any | Preferred | N/A | SM Fiber | Any | N/A | LC |
| Fiber | Any | Preferred | N/A | MM Fiber | Any | 50µm | LC |
| Fiber | Any | Contain | N/A | MM Fiber | Any | 62.5µm | Any |
| Ethernet | 10Mbps | Special | A,B | >= Cat 5 | 10BASE-T | N/A | RJ45 |
| Ethernet | 10Mbps | Special | C | >= Cat 5 | 10BASE-T | N/A | RJ45 |
| Ethernet | 100Mbps | Special | A, B | >= Cat 5 | 100BASE-T | N/A | RJ45 |
| Ethernet | 100Mbps | Preferred | C | >= Cat 5 | 100BASE-T | N/A | RJ45 |
| Ethernet | 1Gbps | Preferred | A, B, D | >= Cat 5e | 1000BASE-T | N/A | RJ45 |
| Ethernet | 1Gbps | Preferred | A, B, D | MM Fiber | 1000BASE-SX | 50µm OM3/4 | LC |
| Ethernet | 1Gbps | Special | A, B, D | SM Fiber | 1000BASE-LX 1000BASE-EX | N/A | LC |
| Ethernet | 10Gbps | Preferred | A, D | Twinax | 10GSFP+Cu 802.3 | N/A | SFP+ |
| Ethernet | 10Gbps | Preferred | A, D | MM Fiber | 10GBASE-SR | 50µm OM3/4 | LC |
| Ethernet | 10Gbps | Special | A, D | SM Fiber | 10GBASE-LR 10GBASE-ER | N/A | LC |
| Ethernet | 40Gbps | Future | A, D | MM Fiber | 40GBASE-SR4 | 50µm OM3/4 | LC |
| Ethernet | 40Gbps | Future | A, D | SM Fiber | 40GBASE-LR4 | N/A | LC |
| Ethernet | 100Gbps | Future | A, D | MM Fiber | 100GBASE-SR10 | 50µm OM3/4 | LC |
| Ethernet | 100Gbps | Future | A, D | SM Fiber | 100GBASE-LR4 | N/A | LC |
| DWDM | 100Gbps | Future | A | SM Fiber | - | N/A | LC |
| CWDM | TBD | Future | TBD | SM Fiber | - | N/A | LC |
| All Others | Any | Special | Any | Any | Any | Any | Any |

## Layer 2—Data Link Layer

| Technology | Status |
| --- | --- |
| Ethernet | Preferred |
| Link Aggregation Control Protocol (LACP) | Preferred |
| MPLS | Preferred |
| 802.1Q | Preferred |
| Link Layer Discovery Protocol (LLDP) | Preferred |
| Cisco Discovery Protocol (CDP) | Special |

## Layer 3—Network Layer

| Technology | Status |
| --- | --- |
| IPv4 | Preferred |
| IPv4 – Multicast | Future |
| IPv6 | Future |
| Differentiated Services (DSCP) | Preferred |
| RFC 1918 Private Internets | Contain |
| Network-to-Network Tunnels | Special |

### IPv4 Routing

| Technology | Standard |
| --- | --- |
| OSPF | Special |
| EIGRP | Special |
| Static | Special |
| BGP or MP-BGP | Preferred |
| BGP or MP-BGP | Special |

## DSCP Standards

| Use | DSCP | DS Field | Status |
| --- | --- | --- | --- |
| VoIP Traffic | Expedited Forwarding (EF) | 101 110 | Preferred |
| LMR and 911 Voice | Network Control 1(NC1) | 110 000 | Preferred |
| Network Control Routing Protocol | Network Control 2(NC2) | 111 000 | Preferred |
| Life and Safety | Assured Forwarding (AF31) Mission-Critical Data | 011 010 | Preferred |
| Video | Assured Forwarding(AF41) | 100 010 | Preferred |
| General | Assured Forwarding (AF11) Best Effort | 001 010 | Preferred |

## Layer 3+

### Perimeter Control – Commonwealth Security Zones

Entities wishing to connect to the Commonwealth may not provide interconnections between Security Zones of the Enterprise network.  All non-Commonwealth connectivity and/or traffic must be isolated from all Commonwealth Security Zones. If there is a special business need to provide this functionality (ex: route leaking for device management), the entity must have its sponsoring agency submit the appropriate COPPAR and obtain approval before implementing such a configuration.

### Perimeter Control – Service Provider Network

All service providers shall be responsible for providing any necessary perimeter control among their client networks and management network.  The service provider is solely responsible for any traffic that is permitted to cross between these networks.  Additionally, such activities may cause the service provider to be in violation of this and/or other Commonwealth standards.

## Voice Standards

- Concurrent calls – Status of this capability is dependent on the agencies needs/requirements.  Providers must disclose the status of this capability to the Commonwealth.

- Make Table:

  - ITU Telecommunication Standardization Sector (ITU-T) Standards

    - G.711 – 64kb uncompressed (Preferred, best voice quality)

    - G.729 – 8kb compressed – high complex algorithm (Special – bandwidth constraints)

    - G.729A – 8kb compressed – medium complexity algorithm – lower quality (Special – bandwidth constraints)

  - T.38 – for FAX – If agency wants total IP configuration

- Session Border Controller (SBC) – A SBC, or gateway device with equivalent functionality, is essential at the agency level to connect to the providers IP/PSTN SIP cloud.  This is required for both new PIP and existing PIP.  ??Does EISO have any requirements for the minimum firewall capabilities of this??

- Encryption is Preferred for all voice traffic on the Commonwealth network.  This ensures compliance with HIPPA, IRS, and other regulatory requirements.  Non-encrypted communications are acceptable for certain application—including those with communications that ultimately are broadcasted in the clear (ex: Fire/Police radio that would not be encrypted otherwise).

## Video Standards

### Video Conferencing

- PRI - 23B+D, FAS signaling, ISDN Protocol NI2, Pulse Type DTMF, 10 digits out, Hunt Sequential Ascending, B Channel configuration 2 Way universal

### General Video Transmission

- Transport stream encapsulation of:

Video - MPEG-2, H.264, H.265 (future)

Audio - MPEG-1, MPEG-2, AAC, Dolby/AC3

- Unicast for point to point transmission

- Multicast for point to multi-point/broadcast transmission

- RTMP/RTSP/HLS stream encapsulation allowed for non-broadcast/web applications using same audio/video standards above

# Facilities

## Environmental Specifications

Entities wishing to house terminating equipment within a Commonwealth Point of Presence (CPOP) facility must submit a request to the Commonwealth, including the following environmental specifications:

- Total power consumption of all equipment

- Power diversity requirements

- Quantity and connector type for all powered devices

- Heat dissipation specifications for all equipment

- Number of rack units (RUs) required for equipment

- Weight of equipment/rack

- Fiber/Copper connectivity required

- Phone/Video/CATV connectivity required

- Surveillance requirements

- Physical security requirements

- Special audit requirements

All equipment must meet the following specifications.  If no device that meets the specifications below is available, the entity must obtain prior approval for a waiver.

- Power

  - 220V AC Preferred, 110V AC as required at Group A and B CPOP locations

  - -48V DC Preferred, 110V AC as required at Group C locations

  - Vendor-owned UPS’s will not be permitted when facility power is already UPS supported

- Proper grounding must be maintained on all equipment.

- Equipment must be rack mounted.

  - Commonwealth-provided racks will be 19” wide

  - If waiver is obtained, the following additional specifications must be provided:

    - Equipment footprint

    - Service area

    - Tile cut-out positioning

    - Full equipment dimensions

## Access to Facilities

### Monitoring and Surveillance

All entities within CPOP locations must support onsite and remote auditing of the CPOP, equipment in the CPOP, and access to the equipment.  This may include alerts from, real-time system access to, or reporting from security, physical access, environmental monitoring, surveillance, and other monitoring systems and may include onsite visits to audit.

### Physical Access

All CPOP facility access will be coordinated through the Commonwealth’s representative.

Voice Only Standard: This standard is Preferred for new PIP Voice circuit installs.

Voice Only Standard: This standard is Preferred for new PIP Voice circuit installs.

Examples:  cable modems, CATV, satellite, wireless, WiFi, microwave, etc.

This standard is also Preferred for PIP SIP trunking.

The Commonwealth’s standard Network Layer protocol is IPv4.  At this time, the Commonwealth does not provide native transport or translation of IPv6 traffic.  The Commonwealth is investigating implementation strategies for IPv6 and will support it in the future.  Vendors must be prepared to support IPv6 when the Commonwealth deploys.

Use of DSCP marking on all Layer 3 traffic is “Preferred”, with standards that prioritize public-safety voice and honor DSCP through disparate networks. If providers do not honor QoS tags, they must, at minimum, pass the QoS tags through their network.  Providers that do not honor the QoS tags must disclose this to the Commonwealth.

To promote interoperability, the Commonwealth will route Commonwealth-assigned RFC1918 prefixes only and any other use is “Contained.”  IPv4 public addresses are “Preferred” when connecting Layer 3 networks.

Private AS numbers must be assigned by the Commonwealth.

Private AS numbers not assigned by the Commonwealth.

Examples:  Building fire alarms, Building security devices, etc.

All data not previously tagged with DSCP should be treated as Bulk Data.