Contract No. PGC-95-12-01


TECHNICAL SPECIFICATION SECTION 9 – FARM GATE
9.1 – SCOPE 

The work included in this section consists of installing a new farm gate across the existing access road.  The new gate shall be installed before any improvements begin on the access road so unauthorized traffic can be controlled.
9.2 – MATERIALS

A. Gate – The gate shall be constructed of minimum 19 gauge steel tubing.  The stell tubing shall be 1-3/4” diameter.  The overall gate dimensions are 50-inches high and 12-feet long.  The gate shall be painted to resist corrosion.  Hinges and snap chains shall be provided with the gate.
B. Posts – Two posts shall be provided for the gate.  The posts shall be pressure treated 6” x 6” wood posts 8-feet long.  The pressure treatment shall be ACQ, MCA or MCQ formulations ground contact concentration. 
9.3 – PROCEDURE
Install the gate at the end of the line of pine trees beyond section 1 of the access road.  Auger drill 12-inch diameter holes 4-feet deep on each side of the road for the wood posts.  Place the posts in the holes and backfill in 6-inch lifts to the ground surface.  Compact each lift by tamping.  Brace and hold the posts plumb during backfill and compaction operations.  Install the gate hinges on the post on the east side of the road.  Mount the gate on the hinges and adjust as necessary so that gate swings open level.  
9.4 – MEASUREMENT AND PAYMENT
Lump Sum.  
TS-9-1

