COMMONWEALTH of PENNSYLVANIA

DEPARTMENT of ENVIRONMENTAL PROTECTION

Division of Air Quality Monitoring
12th Floor
Rachel Carson State Office Building
P.O. Box 8468
Harrisburg, PA 17105-8468

Specifications for Mobile Meteorological Trailer. 
NO SUBSTITUTION PROCUREMENT ON “ALUMA” MANUFACTURED PART ITEMS

| Item Description | Specifications |
| --- | --- |
| ALUMA Trailer Tower (TM-12/T-DD-35HN) ALUMA Guy Wire Kit (2-17CGA) ALUMA Rotor Antenna (YAESU G-800DXA) ALUMA Thrust Bearing, B Section, ASSY ALUMA Winch, 120VAC, 1000LB, W/ Pend ANT Remote. ALUMA Antenna Standoff, Double, 3FT Arms ALUMA Tire, Spare Cabinet, Radio PACKAGE, Electrical REEL, Cord ENCLOSURE, Cables ASSY, JACK, Tongue SHIPPING | Trailer Specifications GALVANIZED STEEL CHASSIS, 4 ’W X 16’L DECK WITH A-FRAME/TBAR TONGUE, 2 WHEEL ELECTRIC BRAKES WITH BREAKAWAY KIT, 5,200LB TORSION AXLE (GVWR 5,190LBS), ST205/75R15 RADIAL TIRES, GALVANIZED WHEELS, FENDERS, 4-HOLE COUPLER CHANNEL WITH 2-5/16 BALL COUPLER, 7,000LB TONGUE JACK, AND ALUMINUM TREADBRITE DECKING. CONTAINS (5) SLIDEOUT OUTRIGGERS WITH (5) 7 ,000LB LEVELING JACKS, W/ (6) FOOT PLATES. INCLUDES LED DOT LIGHTING, 7 PIN RV TYPE TRAILER CONNECTOR, SAFETY CHAINS, AND FLASHING AMBER EMERGENCY LIGHT. COMPLETE WITH MANUAL TILT WINCH, ALUMINUM STORAGE BOX (664M), TRAILER TOOL KIT (TM-TK), AND TOWER GROUNDING KIT (12-8GR). NESTED TOWER (T-DD-35HN), 34FT (10M), TELESCOPING ALUMINUM, CRANK-UP, CONSISTING OF 2-17FT SECTIONS (B,C) WITH 4FT OF OVERLAP. COMPLETE WITH 2”OD X 8 FT L MAST WITH COMBO ROTOR LOWER PLATE, PLASTIC BUSHING TOP PLATE, AND WORM GEAR WINCH. OVERALL TRANSPORT SIZE: 17’11”H X 5’9”W X 8’9”H. APPROX. SHIP WEIGHT: 1,670LBS . GUYED- WL: 70MPH, SA: 18 SQ FT, PL: 125LBS Guy Wire Kit 2-17F T SECTIONS, 3/16 GALV. CABLE ASSY, (3 PER SECTION) Antenna Rotor YAESU G-800DXA WITH CONTROL BOX & 150FT L CORD. MOUNTED/INSTALLED. Thrust Bearing B SECTION, ASS Y, INCLUDES BEARING, TOP PLATE & HARDWARE. MOUNTED/INSTALLED. Winch WINCH, 120VAC, 1000LB, W/ PEND ANT REMOTE, ALUM COVER, ASSY. MOUNTED/INSTALLED. ANTENNA STANDOFF DOUBLE, 3FT ARMS, TOP MOUNT, B HEAVY WALL SECTION, INCLUDES DETENT PINS. NOTE: DOUBLE STANDOFF 16" ARM - TO BE 1.34" OD RATHER THAN STANDARD 2" OD. SPARE TIRE TIRE, SPARE, ST225/75R15, LOAD RANGE E, MOUNTED & LOCKE CABINET 50"Hx30"Wx34"D, SINGLE BAY, CREAM, FOUR POST MOUNTING SHELVES, 120V DUAL FAN KIT, ½” INSULATION, ASSY. NOTE: LED LIGHTING INTERIOR CABINET. MOUNTED AND INSTALLED ON TO TRAILER. ELECTRICAL 30A, 120/240VAC, INCL: SHORE POWER INLET, 100A LOAD CENTER, NEMA3R, (x2) DUPLEX RECEPTACLE/ (X2) GFCI ON (X2) 2 GANG BOX, (X2) REAR GFCI RECEPTACLE ON (X2) SINGLE GANG BOX W/ MOUNTING PANEL. MOUNTED AND INSTALLED TO THE CABINET/TRAILER. CORD REEL REELCRAFT LNM450 CORD REEL, AWG 12/3 X 50. ASSY, MOUNTED AND INSTALLED. ENCLOSURE 14x12x5.5” NEMA4X, HINGE STYLE, WEATHER RESISTANT, -SC 1 CONDUIT FOR CABLES, WHITE, FIBERGLASS-REINFORCED. MOUNTED AND INSTALLED TO CABINET. JACK, TONGUE, SWIVEL WHEEL ASSY, JACK, TONGUE, 1500LB, SWIVEL WHEEL, BOLT-ON. MOUNTED AND INSTALLED. SHIPPING INCLUDE COST OF SHIPPING COMPLETED TRAILER TO: DEP, BUREAU OF LABORATORIES, 2575 INTERSTATE DRIVE, HARRISBURG, PA 17110-9332. |

1. Instructions to Bidders

a. The Department of Environmental Protection (DEP), Bureau of Office Services, is requesting price bids for the following items listed in the Invitation for Bid (IFB) item list section. The bidder must supply a price for all items. Award will be made based on the lowest grand total price.

2. Delivery

a. Physical delivery location: DEP, Bureau of Laboratories, 2575 Interstate Drive, Harrisburg, PA 17110-9332, ATTN: Tim Matzik.

b. Please include all costs including shipping/handling within the costs of the line items supplied, i.e. bid prices must be FOB destination. No additional costs will be allowed.

3. Agency Contacts

a. Any technical questions should be directed to Tim Matzik, phone: 412-523-2951 email: tmatzik@pa.gov. Email communication is preferred.

b. Questions regarding bidding procedures should be directed to Quiana Brown, phone: 717-783-0486, email: qbrown@pa.gov. Email communication is preferred.

# 4. Bid Submission

  - All Bids must be submitted electronically through the Department of General Services’ (DGS) eMarketplace website. The eMarketplace website is located at http://www.emarketplace.state.pa.us. DEP is not responsible for the maintenance of the eMarketplace website.

  - DGS’s Supplier Service Center (https://www.dgs.pa.gov/Materials-Services-Procurement/Supplier-Service-Center/ Pages/default.aspx) is available to assist vendors with registration, bidding, and account management. For questions regarding registration help, send an email to RA-PSC_Supplier_Requests@pa.gov or call (877) 435-7363, choose option 1. For questions regarding bidding help, send an email to srmhelp@pa.gov or call (877) 435-7363, choose option 2.

  - DEP is not responsible for the support or functionality of any Department of General Services website.

5. Estimated Quantities

a. The quantities contained in the IFB section are estimated only and may increase or decrease dependent upon the needs of DEP. Ancillary items may be added to this acquisition (purchase order) at the discretion of both parties at a mutually agreed upon price.

b. Payment to the contractor shall be made within approximately 30 business days after invoice receipt at the unit bid price for the quantities supplied.

6. Requirements

a. See (page 1-2 of this specification) item list section for listing of parts.

b. ALUMA is the requested manufacturer for all items beginning with “ALUMA” under Item Description. Substitution responses will not be accepted.

7. Award

a. Award will be made to the responsible responsive bidder based on the lowest grand total price.

b. Bidders are required to complete and return the “Reciprocal Limitations Form” electronically attached to the IFB file. Reference see Title 62 Pa C.S. Section 107.