Specifications - Invitation for Bid 6100064228

ISSUING OFFICE

COMMONWEALTH OF PENNSYLVANIA DEPARTMENT OF GENERAL SERVICES BUREAU OF PROCUREMENT

1800 Herr Street, 2nd Floor Harrisburg, PA17103

ON BEHALF OF

PENNSYLVANIA GAME COMMISSION

7/28/2026

# PURPOSE:

This Invitation for Bid 61000XXXXX will be to provide and install one 9’ Steel Dump Body on existing 2025 Ford Truck Super Duty F-550 Crew Cab (179” WB, 60” CA) Chassis This vehicle is for the Pennsylvania Game Commission.

# METHOD OF AWARD:

It is the intent of the Commonwealth to make a single award of item(s) listed in the IFB to the lowest responsive and responsible bidder. The Commonwealth Reserves the right to award by line item if it determines that it is in the best interest of the Commonwealth to do so.

# ISSUING OFFICE:

Department of General Services

Bureau of Procurement

1800 Herr Street, 2nd Floor

Harrisburg, PA 17103

Dan Covell, Commodity Specialist

Phone: 717-346-3828    Email: dcovell@pa.gov

# 4. SPECIFICATIONS:

Existing Chassis Specifications

- Manufacturer Window Sticker Included

- 2025 Ford Truck Super Duty F-550 Crew Cab (179” WB, 60” CA) XL Chassis Cab

- Engine: 6.7L Powerstroke Turbo Diesel

- Transmission: 10-Speed TorqueShift Automatic

- GVWR: 19,500-lbs Payload Package

Steel Dump Body Specifications

- Minimum 9’3” length, 94” width, 2-3 cu yd capacity for 60 CA

- Minimum 12” double panel fold down side wall with 10 ga inner panel and 12 ga outer panel

- Minimum 5” longitudinal construction with 3” crossmembers on 12” centers

- Minimum 10 ga floor

- Minimum 1/4 cabshield with window

- Minimum 18” rear height with full height corner posts

- Minimum 10 ga double-acting tailgate with center coal door

- Features boxed top rail and sloped, dirt-shedding, bottom rail on both sides and tailgate

- 6” Sideboard pockets at front and rear

- Factory black finish

- 11-ton double-acting electric hydraulic scissor hoist subframe mounted with pendant control in cab

- LED clearance and marker lights to meet current FMVSS standards

- LED Stop/Tail/Turn recessed in rear corner posts

- Back up camera to provide driver unobstructed view behind vehicle when in reverse operation

- Towing Package to include:

    - Minimum ½” Tow Plate

    - Class V rated 2-1/2” Receiver

    - D-Rings

    - 7-wire trailer plug

- Tarp system mounted on cab shield featuring:

    - 7” folding crank with detent

    - Tarp roller

    - 7’ x 15’ PVC-coated vinyl mesh tarp

    - Rear bar w/20’ pull rope

- Steel pull out step for operator safety mounted streetside

- Grab handle mounted on cab shield on streetside

- Minimum 24 x 18 x 18 steel locking underbody toolbox mounted curbside

- Steel splash shields ahead of rear wheels

- Rear mudflaps with anti-sail device

- Body up light in cab

- Body prop

- Back up alarm