STATEMENT OF WORK 
Bleaching Services

I. SCOPE OF WORK:

The Department of Corrections requires a contractor to provide bleaching, scouring, softening, drying, and anti-curling treatment to un-finished materials used by Correctional Industries located at the State Correctional Institute (SCI) at Phoenix, 1200 Mokychic Drive, Collegeville PA 19426. 
Questions regarding the technical aspect of this bid should be directed to Gerald Arasin at (610) 409-7890 ext. 1456.  Questions regarding the bidding or contracting procedures should be directed to Christina Zambotti (717) 728-3509
II.
CONTRACT TASKS:
The following tasks are to be completed on the un-finished materials under the performance of this contract:  
A.
Scouring Services:
Winter Weight Rib Knit Tubular Cloth, 110,000 Pounds:

1. Scour Winter Weight instead of bleach, No Optical, No Anti-Curl.  Finish to width + ½” -0 and weight spec. +/- 3%.

2. Cut out seams, flat-fold, wrap in plastic, and label with Cloth Style, Size, Lot#  and finish weight lbs., each individual roll of cloth.

3. Place full lot on 1 pallet, max 50” high, strap and shrink wrap pallet for shipping FOB Phoenix.
B.
Bleaching Services:
Summer Weight Rib Knit Tubular Cloth, 145,000 Pounds:

1. Bleach snow white with optical brightener. Finish to width + ½” -0 and weight spec. +/- 3%.

2. Cut out seams, flat-fold, wrap in plastic, and label with Cloth Style, Size, Lot#  and finish weight lbs., each individual roll of cloth.

3. Place full lot on 1 pallet, max 50” high, strap and shrink wrap pallet for shipping FOB Phoenix.
Jersey Flat Knit Tubular Cloth, 295,000 Pounds:

1. Bleach snow white with optical brightener. Finish with anti-curl to width + ½” -0 and weight spec. +/- 3%.

2. Cut out seams, flat-fold, wrap in plastic, and label with Cloth Style, Size, Lot#  and finish weight lbs., each individual roll of cloth.

3. Place full lot on 1 pallet, max 50” high, strap and shrink wrap pallet for shipping FOB Phoenix.

Pick up of Un-finished Materials: Contactor must sign for un-finished materials picked up and verified by SCI Phoenix supervisor, foreman, or manager. 
Return of Finished Materials:  All finished materials must be returned within four (4) weeks of pickup.  SCI Phoenix will weigh all the materials and conduct spot check on all returned materials.  All returned materials are subject to SCI Phoenix supervisor and foreman’s inspections.

III.
CONTRACT REQUIREMENTS:
The contractor shall abide by all Department of Corrections rules and regulations while on State Property Attachment 2 - Security Clearance Information for Vendors and Attachment 3 - General Search Procedures for Vehicles.
IV.
BID AWARD:   
The bidder shall complete the electronic Invitation for Bid found at www.pasupplierportal.state.pa.us and complete and attach the following to the electronic Invitation for Bid, for award of the bid:
Attachment 3 – Anti-Sweatshop Certification.

Attachment 4 - Workers Protection and Investment Certification Form

Bid will be awarded based on the unit price per pound.  The unit price shall include all services required including pickup and delivery of the materials.  The contract quantities are estimated and may increase or decrease, depending on the needs of the Department. If service usage exceeds the original estimated quantities, Vendor agrees to provide additional service at the original contract price up to 20% over the original quantity.
V.
CONTRACT TERM:
The contract shall commence upon execution and receipt of Purchase Order and Notice to Proceed on July 1, 2022, and terminate June 30, 2023.  
Further, the parties may agree to renew this contract for up to four (4) additional consecutive annual terms, with a final termination date of June 30, 2027, upon the same terms and conditions set forth in the contract.  Upon renewal, the contractor may negotiate an increase in the unit price by a rate not to exceed 3%.
VI.
PAYMENT TERMS:
Payments shall be made upon the satisfactory completion of delivery of each lot upon approval of the Department of Corrections staff.
VII.
INVOICES:
All invoices for this contract MUST be sent to the following address:

Commonwealth of PA
PO Invoice

P.O. Box 69180
Harrisburg, PA  17106
All invoices MUST have the Purchase Order Number and Vendor’s SAP Vendor Number on the invoice.  Failure to provide this information could result in delay of payment.

VIII.   BID RESULTS:
Bids will be opened and tabulated on the date and time specified in the invitation for bid. Bid tabulations are available by accessing www.emarketplace.state.pa.us on the day following the bid opening. The bid tabulations are for information only and do NOT constitute actual award/execution of a contract. The results are of the apparent bidders and all bids are under review until final award of the purchase order.

