Contract No. PGC-95-12-01

TECHNICAL SPECIFICATION SECTION 1 - SUMMARY OF WORK
1.1 – SCOPE OF PROJECT

The intent of this project is to gain access to and plug a leaking oil well on State Game Lands (SGL) #95, Parker and Washington Townships, Butler County.  The project consists of improving an existing access road, installing two temporary stream crossings on that access road, plugging the oil well with cement grout, salvaging metal pipes, completing the required paperwork, installing a farm gate and restoring the site.
1.2 – WORK AREA
The oil well is located on SGL #95.  The area is completely wooded and there are two tributary streams to Silver Creek near the oil well.  Most of the existing access road is located on private property.  The EPA has gained permission from the property owners to use the road for this project.
1.3 – WORK HOURS

The work hours at the project site are during regular PGC business hours which are Monday through Friday, 7:00AM to 4:00PM.  Work during different hours must have prior written approval by the PGC.  Requests for different working hours must be submitted in writing two days in advance.  The PGC has no objections to completing in progress operations beyond the scheduled work hours.
1.4 – ACCESS TO WORK AREA
Access to the work area will be on an unimproved access road which connects to Bruin Road (SR 1004).  The current access road has ruts, potholes and trees and brush encroaching on the travel path.  The contractor is required to improve the access road according to Section 5 of the Technical Specifications.  The contractor will be provided a key for the existing locked gate across the access road.  The contractor is responsible for controlling the access road by keeping the gate locked during non-work hours and controlling traffic during work hours.
1.5 – PERMITS, LAWS AND REGULATIONS 

The Contractor shall procure and pay for all permits, licenses, inspections, conveniences, or other approvals necessary for the execution of the contract.  

The Contractor shall comply with all laws, ordinances, rules, orders and regulations relating to the performance of the work, the protection of adjacent property, the maintaining of surface passageways, guard fences, and/or other protective facilities.

All applicable Federal and State laws and regulations, municipal ordinances and rules and regulations of all authorities, having jurisdiction over construction of the project shall apply to the contract throughout, and they shall be deemed to be included in the contract as a part, thereof, the same as though herein written out in full.

All regulations of the Occupational Safety and Health Act are in effect on this contract.  It will be the Contractor’s responsibility to make himself aware of all appropriate County, State and Federal regulations that apply to this contract.

Any violations incurred from improper execution of the above provisions shall be paid for by the Contractor.  Loss of time on the project from such violations will not be tolerated. 

TS-1-1

