Contract No. PGC-95-12-01

TECHNICAL SPECIFICATION SECTION 3 – CLEARING AND BRUSHING
3.1 -SCOPE
This work is removal and disposal of all trees and brush, fallen and standing in the access road or around the oil well that would interfere with project operations.  Clearing is removal of an entire tree or bush down to the stump.  Brushing is selected trimming of limbs on trees and bushes.     
3.2 - PROCEDURE

Before commencing clearing and brushing operations, mark the limits at the site.  The limits of clearing and brushing must be reviewed and approved by the PGC representative at the site.  There should be very little to no clearing and brushing necessary on the private land along the access road.  Do not damage any plants or trees, natural growth, or other objects outside the areas to be cleared and grubbed.
Cut down the trees and brush within the approved limits.  Cut the trees and brush so that the stumps are no more than three inches above the ground.  Stumps can remain.  Grade as necessary around the stumps to allow for safe passage of vehicles and equipment.  Brush, branches, limbs and logs are to be cut into manageable sections and removed from the access road and well areas.  The PGC will designate an area on state game lands property for disposal of the cleared and brushed material.  Burning is not permitted at the site.
3.3 - MEASUREMENT AND PAYMENT
Lump Sum.
TS-3-1


